Chip back potential is the level which bulk silicon is maintained by on-chip connection, or it is the level to which the chip back must be connected when specifically stated below. If no potential is given the chip back should be isolated.

.052”

PAD FUNCTIONS:

2	S3

3	D3

4	D1

5	S1

6	IN1

7	GND

8	V-

9	IN2

10	S2

11	D2

12	D4

13	S4

14	V+ (Substrate)

.070”

Top Material: Al

Backside Material: Si

Bond Pad Size:  .004” X .004”

Backside Potential: V-

Mask Ref: B

APPROVED BY: DK 			DIE SIZE .052” X .070” 	                 DATE:  11/22/16

MFG: SILICONIX		                	   THICKNESS .014”                       	         P/N: DG303A

DG 10.1.2

#### Rev B, 7/19/02